Binary Ripple Counter

- Follows binary sequence

- Consists of a series of flip flops, with the output of each flip flop connected to the clock pulse input of the next flip flop

- Flip flop of least significant bit receives main clock pulse

- Uses negative edge triggers

- Frequency of clock pulse input for each flip flop is half the frequency of the clock pulse input of the previous flip flop

- Signal propagates through the counter in a ripple fashion

- For n flip flops, the counter counts from 0 to 2n-1, in binary

- Flip flops are not clocked simultaneously, so the counter is asynchronous

- Delay of each flip flop comes into account, making the counter slow

Shift Register

- Register consists of group of n flip flops in series to store n bits of data

- Shift registers are capable of shifting information in one cell to its neighbour by applying clock pulses

- Output of each flip flop acts as the input to the next

- Data can be shifted left to right or right to left or both ways, depending on whether the register is unidirectional or bidirectional

- Multiple forms of shift registers exist, like serial input serial output (SISO), serial input parallel output (SIPO), parallel input serial output (PISO) and parallel input parallel output (PIPO)

- Universal shift registers combine all the different forms of shift registers

Hamming Code

- Block code capable of detecting up to two simultaneous bit errors and correcting single-bit errors

- Source encodes message by inserting parity bits at the positions in the message which are powers of 2, i.e. at 1, 2, 4, 8 etc.

- The parity bits have a value of 1 or 0 depending on whether the number of 1’s in the variables it considers is odd, giving the parity bit a value of 1, or even, giving the parity bit a value of 0

- Destination receives message and performs recalculations to detect errors and find the bit position of the error

Encoding Process

- Calculation of the number of parity bits.

- Positioning the parity bits.

- Calculating the values of each parity bit.

Decoding Process

- Calculation of the number of check bits.

- Evaluating the check bits.

- Error detection and correction