Final DLD Projects titles and descriptions

|  | 2D LED Digit Generator | - Construct a 10x10 or 15x15 LED Square Board that will be operated through multiplexer (16:1) - Based on given decimal number as input to the MUX, the board will show corresponding decimal digit through those LEDs on the board. |
| --- | --- | --- |
|  | 2D LED English Letter Generator | - Construct a 10x10 or 15x15 LED Square Board that will be operated through multiplexer (32:1) - Based on given English character as input to the MUX (assumed by programmer anyway), the board will show corresponding characters through those LEDs on the board. |
|  | LED based 2D matrix traveler | - Use the LED 10x10 matrix to display 2D grid. - A LED light will be turned on at a time, but in this case, each LED will gets chance to be turned on equally for 0.5 sec - Follow a pattern so that all LEDs can turned on sequentially |
|  | 7 segment Digit display | - Use 7 segment display, multiplexer, encoder to turn on the expected digit given as input by user. |
|  | 4 bit binary ADDER Calculator | - User will give two 4 bit binary number - The circuit will be able to add them and show the result through 7 segments display - It will be also able to notify is there any overflow is occurred or not. |

|  | 4 bit binary SUBTRACTOR Calculator | - User will give two 4 bit binary number - The circuit will be able to subtract them and show the result through 7 segments display - It will be also able to notify is there any underflow is occurred or not. - The result should be in regular form where if the number is negative it should show it through negative sign. |
| --- | --- | --- |
|  | 4 bit binary MULTIPLIER Calculator | - User will give two 4 bit binary number - The circuit will be able to multiply them and show the result through 7 segments display - You have to calculate by yourself how many 7 segments display will be required for any random binary number containing at most 4 bits in any number. |
|  | Password based Door Lock System (Circuit breaker) | - In this system, one door (can be designed as red and green LED) will controlled based on binary password. - A 6 bit password will be stored in memory. A user/ person, who wants to access to the room, must enter password through switches. - If the password is matched, the door will be open (Green LED). Otherwise the door will be closed (Red LED) |

|  | Digital Smart Room | - The room should be able to detect and display the number of people inside (you can use two switches to control enter and leave pressing separately. If Switch A is pressed a person will enter and if Switch B is pressed a person will leave. Pressing several times will increase/ decrease the number) - It should keep a track of when someone enters the room and when someone leaves - The number of lights (LED 1) and fans (LED 2) turning on in the room should be proportional to the number of people in |
| --- | --- | --- |
|  | Traffic control system for a cross road | - 4 roads in cross section have 3 lights (Green, yellow and red) each - You have to play a role of a traffic controller in a traditional way so that no accident can be occurred - You can take help from traffic control guide to do this task - You have to keep 4 memories (buffer) for each of road to keep track how many vehicles are there in each road. - Each road will increase its number of vehicles slower than the speed of decreasing the number of vehicles - You are suggested to use multiplexer, clock of different frequency and other common combinational logic circuits |

|  | #### Vending Machine | - A vending machine has different items to sell to the customer having different prices - User can request for different items through different switches - When a product is sold out that particular counter will be decremented - Gradually “total money” counter will be increased based on the item’s price weighted separately. |
| --- | --- | --- |
|  | Pattern/ Sequence Counter | In bits stream you have to count how many times a particular sequence has been occurred That sequence can be stored in a register (a group of flip flops) Each time that sequence occurred, the counter value will be increased by one |
|  | Target Shooting game | - In 3x3 LED matrix, randomly an LED will be turned in that grid, user have to fix the coordinate of that LED with in very short time - User will give the coordinate of that LED through 3x3 switch matrix - If they are matched, user score will be upgraded by one. |
|  | 4 bit Ring counter | - Theory discussed in book - Values in binary should be represented in two 7 segments displays |
|  | 4 bit Jonson counter | - Theory discussed in book - Values in binary should be represented in two 7 segments displays |
|  | Cross road challenge | - A car has 2 head lights and 2 back lights, these will be operated by the user using multiplexer - It has four options – turn left, turn right, go forward and go backward. - Based on these options those head lights and back lights should be turned on following general rules for a vehicle |
|  | Water Tank Control Mechanism | - A random water level will be displayed by a 7 segment display - There should have a certain maximum and minimum water level for that tank - There should have a switch that will decrease the water level based on user input. - If the water level goes beyond minimum, automatically it will increase the water level up to maximum level gradually (using a counter) - If maximum level is reached, automatically it will cut off the supply (just stop the counter and reposition to 0) |
|  | Temperature Controlled Fan | - You are asked to control the speed of the fan based on temperature - Temperature will be controlled by user - Based on temperature fan will speed up or down |
|  | Digital clock | - As per rule |
|  | Stop watch | - As per rule |
|  | Digital Timer | - As per rule - User will set the time which will be stored in a register (group of flip flops) |
|  | Bidirectional blinking light | - In this project, there will be a 10X1 LED matrix - If the switch is on, LEDs will be turning on and off sequentially from one direction to another and after touching the end LED it will be moving back - Thus it will be going back and forth until the switch is off. |
|  | 1 Counter | - From a given time sequence, time can be presented in binary form - In a particular value of time period, 1 and 0 will be appeared - Your task is to count total number of 1s appeared so far and display it in 7 segment display continuously |
|  | Random Number Generator | - You have to generate random number following any easy algorithm where no sequence cannot be made ever within 0 to 31. - That random number should be displayed using two 7 Segment Displays |

|  | Unbiased Digital Dice with LEDs | - Digital dice where the dots are defined by 6 LEDs - You have to make it unbiased so that nobody can guess its value earlier - Each time switch is on, it will show random number of LEDs tured on from 1 to 6 |
| --- | --- | --- |
|  | Multi converter | - Your task is converting given value in binary to any other system based on user choice through multiplexer - You can convert your value to grey, excess 3, 24-2-1 and so on based on your choice |