Chapter 1 – Basic Concepts

Table of Contents

1.1 Introduction	2

1.3 Charge and Current	3

1.4 Voltage	4

1.5 Energy and Power	5

1.6 Circuit Elements	6

1.7 Applications	7

## 1.1 Introduction

An electrical circuit is an inter-connection of electrical elements (voltage source, resistance, conductors etc.). Electrical circuits can be open or closed.

## 1.3 Charge and Current

Charge is an electrical property of the atomic particles of which matter consists. Charge is measured in coulombs (c).

1e=1.6×10-19c

Current is the time rate of flow of charge. It is measured in amperes (A).

i=dqdt		I=Qt

With regards to distance travelled by the charge, if the distance between two points of a connection is l, the total distance travelled will be 2l, since current must also flow back.

Example 1.2

q=5tsin4πt×mc

At t=0.5s, find i.

i=dqdt=ddt5tsin4πt

=5tcos4πt×4π+5sin4πt

=50.54πcos4π(0.5)+5sin4π(0.5)

=10π A

## 1.4 Voltage

Voltage is the energy needed to move a unit charge through a conductor. The unit is volts (V=Jc).

v=dwdq

V=WQ

Vab=Va-Vb

Va – Potential at point a with respect to ground

Vb – Potential at point b with respect to ground

Vba=Vb-Va=-Va-Vb=-Vab

## 1.5 Energy and Power

Energy is the ability to do work. The unit is Joules (J).

Power is the rate at which work is done. The unit is Watt (W).

P=dwdt=dwdq×dqdt=vi

V×A=Jc×cs=Js=W

Sign Convention

+ve power is when energy comes from the positive side and -ve power is when energy comes from the negative side.

Practice Problem 5.5

t=3ms	i=5cos60πt	v=3i

P=vi=3i×i=3×5cos60πt2=75cos60π3100=49.1 W

## 1.6 Circuit Elements

Independent Voltage Source

Independent Current Source

Dependent Voltage Source

Dependent Current Source

## 1.7 Applications

Electric Bill is measured in KWH. The rate changes based on usage.

Example 1.9

700KWH	$12.00+100×$0.16+200×$0.10+400×$0.06=$72.00

Average Rate =$72.00700=$10.2/KWH