Chapter 2: Basic Laws

Table of Contents

2.2 Ohm’s Law	2

2.3 Nodes, Branches, Loops and Meshes	4

2.4 Kirchhoff’s Laws	5

2.5 Series Resistors and Voltage Divisions	6

2.6 Parallel Resistors and Current Divisions	7

2.7 Wye – Delta	9

## 2.2 Ohm’s Law

At constant temperature, the flow of current through a conductor is directly proportional to the potential difference between the two terminals.

I∝V

I=GV 	where G is conductance

I=1RV		where R is resistance

V=IR

R=ρLA		where L is the length of the conductor

A is the cross-sectional area

ρ is the resistivity of the material of the resistor

The unit of resistance is ohm (Ω).

VA=Ω

An open circuit is an incomplete circuit. Since current does not flow, I=0, meaning R=∞.

A short circuit is when there is no potential difference between two points. Here R=0.

Power,	P=VI

P=I2R

P=V2R

Practice Problem 2.3

P=20cos2t mW

v=10cost

i=Pv=20cos2t×10-310cost=2×10-3cost A

R=vi=5 kΩ

## 2.3 Nodes, Branches, Loops and Meshes

A branch represents a single element, like a voltage source.

A node is the point of connection in between two or more branches.

A loop is any closed path in a circuit.

A mesh is a non-overlapping loop.

In the diagram, there are:

3 nodes (labelled in diagram)

5 branches

6 loops

3 meshes

In general,	b=m+n-1

## 2.4 Kirchhoff’s Laws

Kirchhoff’s Current Law (K.C.L.)

The current entering a node is equal to the current leaving it.

Kirchhoff’s Voltage Law (K.V.L.)

In a closed loop, the total voltage rise is equal to the total voltage drop.

Practice Problem 2.7

9=i0+i04+v08

10i0+v0=72

We also know,	i0=v02

∴6v0=72

v0=12 V

i0=6 A

## 2.5 Series Resistors and Voltage Divisions

For series connections, the end node of one branch should be the starting node of another branch, and there should not be any other multiple paths in between.

The current in a series connection is the same, since there is only one path.

V1=IR1	V2=IR2

V1+V2=VT

IR1+IR2=IRT

RT=R1+R2

Voltage Divisions:

I=VTRT

V1=IR1=VTRT×R1

The ratio of total voltage to total resistance is equal to the ratio of voltage across a single component to the resistance of that component, for series connections. This is known as the voltage divisor rule.

## 2.6 Parallel Resistors and Current Divisions

For parallel connections, the starting and ending nodes of two or more branches should be the same and there should not be any other multiple paths in between.

Different paths will cause different rates of flow of current. This means current in parallel paths are different, but voltages are the same.

I1=VR1		I2=VR2

IT=I1+I2

VRT=VR1+VR2

1RT=1R1+1R2

Current Divisions:

V=I1R1=ITRT

I1=ITRTR1		I2=ITR2R1+R2

This is known as the current divisor rule.

For conductance (G), the equations are flipped.

For series connections, 1GT=1G1+1G2.

For parallel connections, GT=G1+G2.

Between a and b, there is 1 branch. Between b and c there are 3 branches. The total resistance is the same as that between nodes a and b.

## 2.7 Wye – Delta

Delta Connection		Wye Connection

R13∆=Rb||Ra+Rc=1Rb+1Ra+Rc-1

R13Y=R1+R3

R1+R3=RaRb+RbRCRa+Rb+Rc

R2+R3=RaRc+RaRbRa+Rb+Rc

R1+R2=RaRc+RbRcRa+Rb+Rc

R1=RbRcRa+Rb+Rc

R2=RaRcRa+Rb+Rc

R3=RaRbRa+Rb+Rc

Ra=R1R2+R2R3+R1R3R1

Rb=R1R2+R2R3+R1R3R2

Rc=R1R2+R2R3+R1R3R3

Converting from Delta to Wye or vice versa can make circuits easier to solve.