# ISLAMIC UNIVERSITY OF TECHNOLOGY

# Organization of Islamic Cooperation

# Board Bazar, Gazipur

# Lab 05

# EEE 4484

# Safayat Bin Hakim

Full-Adder Circuit

Source Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY full_adder IS
    PORT (
        Num1, Num2, CarryIn : IN std_logic;
        Sum, CarryOut : OUT std_logic
    );
END full_adder;

ARCHITECTURE behavorial OF full_adder IS
BEGIN
    Sum <= Num1 XOR Num2 XOR CarryIn;
    CarryOut <= (Num1 AND Num2) OR (CarryIn AND Num1) OR (CarryIn AND Num2);
END behavorial;

VHDL

Testbench Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_full_adder IS
END tb_full_adder;

ARCHITECTURE behavior OF tb_full_adder IS
    COMPONENT full_adder
        PORT (
            Num1, Num2, CarryIn : IN std_logic;
            Sum, CarryOut : OUT std_logic
        );
    END COMPONENT;

    SIGNAL tb_Num1, tb_Num2, tb_CarryIn : std_logic := '0';
    SIGNAL tb_Sum, tb_CarryOut : std_logic;

BEGIN
    uut : full_adder
    PORT MAP(
        Num1 => tb_Num1, 
        Num2 => tb_Num2, 
        CarryIn => tb_CarryIn, 
        Sum => tb_Sum, 
        CarryOut => tb_CarryOut
    );

    stim_proc : PROCESS
    BEGIN
        WAIT FOR 100 ns;

        tb_Num1 <= '1';
        tb_Num2 <= '0';
        tb_CarryIn <= '0';
        WAIT FOR 50 ns;

        tb_Num1 <= '0';
        tb_Num2 <= '1';
        tb_CarryIn <= '0';
        WAIT FOR 50 ns;

        tb_Num1 <= '1';
        tb_Num2 <= '1';
        tb_CarryIn <= '0';
        WAIT FOR 50 ns;

        tb_Num1 <= '0';
        tb_Num2 <= '0';
        tb_CarryIn <= '1';
        WAIT FOR 50 ns;

        tb_Num1 <= '1';
        tb_Num2 <= '0';
        tb_CarryIn <= '1';
        WAIT FOR 50 ns;

        tb_Num1 <= '0';
        tb_Num2 <= '1';
        tb_CarryIn <= '1';
        WAIT FOR 50 ns;

tb_Num1 <= '1';
        tb_Num2 <= '1';
        tb_CarryIn <= '1';
        WAIT FOR 50 ns;

    END PROCESS;
END;

VHDL

Waveshape:

1-to-8 Demultiplexer

Source Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY demux_1_8 IS
    PORT (
   input : IN std_logic;
        sel : IN std_logic_vector(2 DOWNTO 0);
        output : OUT std_logic_vector(7 DOWNTO 0)
    );
END ENTITY;

ARCHITECTURE behavioral OF demux_1_8 IS

BEGIN
    PROCESS (input, sel)
    BEGIN
        output <= "00000000";
        
        CASE sel IS
            WHEN "000" => output(0) <= input;
            WHEN "001" => output(1) <= input;
            WHEN "010" => output(2) <= input;
            WHEN "011" => output(3) <= input;
            WHEN "100" => output(4) <= input;
            WHEN "101" => output(5) <= input;
            WHEN "110" => output(6) <= input;
            WHEN "111" => output(7) <= input;
            WHEN OTHERS => output <= "00000000";
        END CASE;
    END PROCESS;
END ARCHITECTURE behavioral;

VHDL

Testbench Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY tb_demux_1_8 IS
END tb_demux_1_8;

ARCHITECTURE behavior OF tb_demux_1_8 IS

    COMPONENT demux_1_8
        PORT (
            input : IN std_logic;
            sel : IN std_logic_vector(2 DOWNTO 0);
            output : OUT std_logic_vector(7 DOWNTO 0)
        );
    END COMPONENT;

    SIGNAL tb_input : std_logic := '0';
    SIGNAL tb_sel : std_logic_vector (2 DOWNTO 0) := (OTHERS => '0');
    SIGNAL tb_output : std_logic_vector (7 DOWNTO 0);

BEGIN
    uut : demux_1_8
    PORT MAP(
        input => tb_input, 
        sel => tb_sel, 
        output => tb_output
    );

    stim_proc : PROCESS
    BEGIN
        WAIT FOR 100 ns;

        tb_input <= '1';
        tb_sel <= "000";
        WAIT FOR 50 ns;

        tb_sel <= "001";
        WAIT FOR 50 ns;

        tb_sel <= "010";
        WAIT FOR 50 ns;

tb_sel <= "011";
        WAIT FOR 50 ns;

        tb_sel <= "100";
        WAIT FOR 50 ns;

        tb_sel <= "101";
        WAIT FOR 50 ns;

        tb_sel <= "110";
        WAIT FOR 50 ns;

        tb_sel <= "111";
        WAIT FOR 50 ns;
      
        WAIT;
    END PROCESS;
END;

VHDL

Waveshape:

4-Bit ALU

Source Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;
USE ieee.numeric_std.ALL;

ENTITY alu_4_bit IS
    PORT (
        num1, num2 : IN signed(3 DOWNTO 0);
        sel : IN std_logic_vector (1 DOWNTO 0);
        result : OUT signed(3 DOWNTO 0)
    );
END alu_4_bit;

ARCHITECTURE behavioral OF alu_4_bit IS
BEGIN
    PROCESS (num1, num2, sel)
    BEGIN
        
        CASE sel IS
            WHEN "00" => result <= num1 + num2;
            WHEN "01" => result <= num1 - num2;
            WHEN "10" => result <= num1 AND num2;
            WHEN "11" => result <= num1 OR num2;
            WHEN OTHERS => NULL;
        END CASE;
 
    END PROCESS;

END behavioral;

VHDL

Testbench Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;
USE ieee.numeric_std.ALL;
 
ENTITY tb_alu_4_bit IS
END tb_alu_4_bit;
 
ARCHITECTURE behavioral OF tb_alu_4_bit IS
 
    COMPONENT alu_4_bit
        PORT (
            num1, num2 : IN signed(3 DOWNTO 0);
            sel : IN std_logic_vector(1 DOWNTO 0);
            result: OUT signed(3 DOWNTO 0)
        );
    END COMPONENT;

    SIGNAL tb_num1, tb_num2 : signed(3 DOWNTO 0) := (OTHERS => '0');
    SIGNAL tb_sel : std_logic_vector(1 DOWNTO 0) := (OTHERS => '0');
    SIGNAL tb_result : signed(3 DOWNTO 0);
 
BEGIN
    uut : alu_4_bit
    PORT MAP(
        num1 => tb_num1, 
        num2 => tb_num2, 
        sel => tb_sel, 
        result => tb_result
    );

    stim_proc : PROCESS
    BEGIN
        WAIT FOR 100 ns;

        tb_num1 <= "1001";
        tb_num2 <= "1111";
        tb_sel <= "00";
        WAIT FOR 50 ns;

        tb_sel <= "01";
        WAIT FOR 50 ns;

tb_sel <= "10";
        WAIT FOR 50 ns;

        tb_sel <= "11";
    END PROCESS;
END behavioral;

VHDL

Waveshape:

Synchronous 5-Bit Shift Register

Source Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;

ENTITY shift_reg_5_bit IS
    PORT (
        input : IN STD_LOGIC_VECTOR (4 DOWNTO 0);
        clock : IN STD_LOGIC;
        output : OUT STD_LOGIC_VECTOR (4 DOWNTO 0)
    );
END ENTITY shift_reg_5_bit;

ARCHITECTURE behavioral OF shift_reg_5_bit IS
BEGIN
    PROCESS (clock, input)
    BEGIN
        IF (clock'EVENT AND clock = '1') THEN
            output <= input;
        END IF;
    END PROCESS;
END behavioral;

VHDL

Testbench Code:

LIBRARY ieee;
USE ieee.std_logic_1164.ALL;
USE ieee.numeric_std.ALL;

ENTITY tb_shift_reg_5_bit IS
END ENTITY tb_shift_reg_5_bit;

ARCHITECTURE behavioral OF tb_shift_reg_5_bit IS
    COMPONENT shift_reg_5_bit
        PORT (
            input : IN STD_LOGIC_VECTOR (4 DOWNTO 0);
            clock : IN STD_LOGIC;
            output : OUT STD_LOGIC_VECTOR (4 DOWNTO 0)
        );
    END COMPONENT shift_reg_5_bit;
    
    SIGNAL tb_input : STD_LOGIC_VECTOR (4 DOWNTO 0) := (OTHERS => '0');
    SIGNAL tb_clock : STD_LOGIC := '0';
    SIGNAL tb_output : STD_LOGIC_VECTOR (4 DOWNTO 0);
    CONSTANT clock_period : TIME := 100 ns;

BEGIN
    uut : shift_reg_5_bit
    PORT MAP(
        input => tb_input, 
        clock => tb_clock, 
        output => tb_output
    );
   
    clk_process : PROCESS
    BEGIN
        tb_clock <= '0';
        WAIT FOR clock_period/2;
        tb_clock <= '1';
        WAIT FOR clock_period/2;
    END PROCESS;
   
    stim_process : PROCESS
    BEGIN
        WAIT FOR 100 ns;

        tb_input <= "00001";
        WAIT FOR 100 ns;

        tb_input <= "00010";
        WAIT FOR 100 ns;

        tb_input <= "00100";
        WAIT FOR 100 ns;

        tb_input <= "01000";
        WAIT FOR 100 ns;


        tb_input <= "10000";
        WAIT FOR 100 ns;

        tb_input <= "10001";
        WAIT FOR 100 ns;

        tb_input <= "10010";
        WAIT FOR 100 ns;

        tb_input <= "10100";
        WAIT FOR 100 ns;

        tb_input <= "11000";
        WAIT FOR 100 ns;

        tb_input <= "11010";
        WAIT FOR 100 ns;

        tb_input <= "11110";
        WAIT FOR 100 ns;

        tb_input <= "11111";
    END PROCESS;
END behavioral;

VHDL

Waveshape: