80286 Microprocessor

Table of Contents

80186	2

Salient Features	3

Virtual Memory	3

Operating Modes	4

Multitasking	5

Memory Management Unit	6

Internal Block Diagram	7

Register Organization	9

Flags	10

Machine Status Word	11

## 80186

We are skipping over the 80186 microprocessor because it is the same as the 8086 microprocessor, just with four additional components:

- Clock Generator – This is a component we have already seen, an IC that generates clock pulses. The 80186 has this built into the IC.

- 2 Independent DMA Channels – We already had one DMA channel, the HOLD and HLDA pins. 80186 allows two such channels (but this could be using the same pair of pins).

- Programmable IC - The 80186 is a programmable IC.

- 3 Programmable 16-bit Timers

The 80186 is more of a controller than a microprocessor. It is used mostly in industrial control applications.

## Salient Features

The Intel 80286 is a high-performance microprocessor with memory management and protection. It is the first member of the family of advanced microprocessors that has on-chip memory management and protection abilities, primarily designed for multi-user or multitasking systems.

The microprocessor is available in 12.5MHz, 10MHz and 8MHz clock frequencies. It is housed in a 68-pin package with 134,000 transistors. These pins are non-multiplexed, meaning there are 24 pins for the address bus and a separate 16 pins for the data bus.

With regards to memory, the 80286 has a 24-bit address bus, which allows addressing of 16MB of physical memory. The data bus is still of 16 bits. If we compare this with previous microprocessors:

### Virtual Memory

80286 also introduces the concept of virtual memory. This is when a program requires more RAM than is available, so space from the secondary storage, the hard disk, is allocated for its use. There is 1GB of virtual memory for each task.

### Operating Modes

The Intel 80286 has two operating modes.

The first is called the Real Address Mode. In this mode, the 80286 is just a fast 8086, up to 6 times faster, with memory management and protection mechanisms disabled. This allows the microprocessor to only address the first 1MB of memory space. This 1MB is called the real memory, conventional memory or DOS memory system. The concepts of segments and offsets come into play here.

Note that, for Real Address Mode, the operations are exactly like 8086, but the memory has 6 bytes per location.

Windows does not use the real address mode since it requires more than 1MB of memory. This brings us to the second operating mode, the Protected Virtual Address Mode. In this mode, the 80286 operates with all its memory management and protection capabilities with the advanced instruction set. Since all 24 address lines are used, the complete 16 MB of physical memory and 1 GB of virtual memory can be used.

### Multitasking

The 80286 includes some special instructions, such as to allow the operating system to perform multitasking. If a new task with a higher priority needs to be performed, the microprocessor does the following:

- End the current task

- Save its states

- Switch to the new task

- Load its states

- Begin executing the new task

This is very similar to how interrupts work. The difference is that interrupts are treated as high priority tasks, and in multitasking, we simply use privilege levels for tasks.

There are four privilege levels, 00, 01, 10 and 11, going from highest priority to lowest.

## Memory Management Unit

The memory management unit allows the 80286 to switch to protected mode and start using virtual memory.

Here, we are no longer using segments, but instead, a 16-bit selector. The selector is used to select a descriptor for the desired segment in a table of descriptors.

Each descriptor describes a 64 KB memory segment, and there are 16,000 descriptors in total, which brings the maximum addressable virtual memory up to 1GB.

The descriptor contains a 24-bit physical address. For the selected segment, a 16-bit offset is also used to access a specific location.

We will look into specific examples of how to use this mechanism later on.

## Internal Block Diagram

The 80286 has four sections, the Execution Unit and the Bus Unit that were present in the 8086 are still present here, but the Address Unit and the Instruction Unit have been added.

The Address Unit deals with deriving physical addresses. This requires a separate unit because of the two operation modes. Again, the exact process of how the address is generated will be discussed later on.

Next comes the Bus Unit, which is responsible for all external communication. The address generated by the Address Unit enters the Address Latches and Drivers section of the Bus Unit. From here, it goes out through the Address Bus. The data is received through the Data Bus and enters the Data Transceivers section.

Notice that the Bus Unit has a 6-Byte Prefetch Queue section. This means that while the microprocessor is executing something, 6 more bytes worth of data have been prefetched for the next instructions. This is why this microprocessor is 6 times faster than the 8086.

From the 6-Byte Prefetch Queue, instructions enter the Instruction Decoder, which is in the Instruction Unit. There is an Instruction Queue here, similar to the one from the 8086. Decoded instructions go to the Execution Unit.

In the Execution Unit, the main parts are the Register Bank, the ALU and the Control Unit. There is an additional special 16-bit register called the Machine Status Word (MSW). The Register Bank and MSW are discussed in depth below.

The Reset, Clock and Power Supplies are connected to the Instruction Unit. Interrupts and Errors are connected directly to the Execution Unit.

## Register Organization

The 80286 has the same registers that the 8086 has:

- 8 16-bit General Purpose Registers – Data registers, Base registers, Pointer registers and Index registers

- 4 16-bit Segment Registers

- Status and Control Registers

- Instruction Pointer

### Flags

The Flag Register has some changes in it though.

The Nested Task (NT) and I/O Privilege Level (IOPL) flags are new.

The IOPL flag uses two bits and defines one of the privilege levels previously mentioned for the current task.

If a task with a higher privilege level arrives while one with a lower privilege level is executing, the old task is suspended and the new task is executed. In this scenario, the new task is called a nested task. The NT flag indicates that the current task is a nested task.

### Machine Status Word

The only new register is the Machine Status Word (MSW).

Even though it has 16 bits, only the lower 4 bits are used as four flags:

- Protection Enable (PE) – This indicates that the microprocessor is running in Protected Virtual Mode.

- Monitor Processor Extension (MP) – Microprocessors can have co-processors, which are connected to the main processor, to assist in some activities. If the co-processor is being monitored, this flag is set.

- Processor Extension Emulator (EM) – Instead of using a co-processor, the microprocessor can divide itself and work in separate parts as though one part were a co-processor. In this scenario, this flag is set.

- Task Switch (TS) – This flag is set if the microprocessor is currently switching from one task to another.