Loops

We use the CX register as the loop counter when dealing with loops. We decrement CX and while it is not 0, a label is used to jump back to a specified location. Otherwise, the code continues from the next line.

ORG 100H
.DATA
.CODE
MAIN PROC
   XOR CX, CX    ; resetting the CX register
   MOV CX, 10    ; initially, CX = 10
   MOV AH, 2     ; single-character output
   MOV DL, '*'
   loopStart:
      INT 21H         ; prints '*'
      LOOP loopStart  ; decrements CX, and goes back to label if it is not 0
   MOV DL, '#'        ; prints '#' at end of loop
   INT 21H
   MOV AH, 4CH     ; equivalent to return statement, returns control to OS
    INT 21H
MAIN ENDP
END MAIN
RET

ASSEMBLY