Procedures

Procedures are like functions that are used in higher level languages. However, we cannot pass arguments to procedures like we can with functions. It is essentially just a separate piece of code. We must call it and when it is finished, we must return to the previous point from where the procedure was called.

The CALL instruction causes:

- The value of the IP register to be incremented to the address of the next instruction and stored in the stack.

- The address of first instruction from the procedure to be set as the value of the IP register.

- Execution to be restarted in the procedure.

The RET instruction causes:

- The old value of the IP register to be restored from the stack.

- Execution to be restarted at the point where the procedure was called.

Example:

ORG 0100H

.DATA
StrArray DB 'Hello World!!$'    ; define string to display

.CODE
MAIN PROC
    MOV AX, @DATA
    MOV DS,AX
    LEA DX, StrArray    ; DX = StrArray[0]
    CALL USER       ; call procedure
    MOV AH, 4CH 
    MOV AL, 00H     ; a code after procedure call and return
    INT 21H         ; exit to DOS
MAIN ENDP

USER PROC       ; declare a procedure named USER
    MOV AH, 09H 
    INT 21H 
    RET         ; return to MAIN procedure
USER ENDP       ; end of procedure USER

END MAIN        ; end of program

ASSEMBLY