IPv6 Configuration

When configuring an IPv6 address, we first need to enable the use of IPv6 on the respective device.

Router(config)#ipv6 unicast-routing

CLI

Next, we can assign an address to a specific interface, similar to how we assigned addresses for IPv4.

Router(config)#interface g0/0/0
Router(config-if)#ipv6 address 2001:DB8:ABCD:1::1/64
Router(config-if)#no shutdown

CLI

When using IPv6, each interface has two addresses, one IPv6 address which is unique globally, and one IPv6 address which is unique locally. The latter is called the link-local address. Link-local addresses always start with the format FE80∷/10. One possible use for them is as a default gateway.

When we assign an IPv6 address to a device, a link-local address is automatically generated. However, we can override that address.

Router(config-if)#ipv6 address FE80::1 link-local

CLI

Notice that, unlike the normal IPv6 address, the CIDR notation has not been used here.

The above configuration was for a router. For PCs, we can simply directly assign the address in the IP Configuration Panel. Note that we can use either the interface address assigned to the router or the link-local address as the default gateway.