Static Routing

Table of Contents

Configuration	3

Default Routes	5

Primary Static Default Route	5

Floating Static Routes	6

Static routing is the process of manually adding entries to a routing table. Any changes to the routing table in the future must also be done manually. This means that the network administrator must know the entire network topology to be able to add the entries, which is difficult and impractical for larger networks. Because of this, on large networks, static routing is only used as a backup system for dynamic routing.

For smaller networks however, static routing provides the benefit of lower resource and bandwidth usage, since no part of the bandwidth is being used to update the routing tables, as is done in dynamic routing.

## Configuration

The command required to specify a static route has the following format:

ip route destination_network_prefix destination_prefix_mask (next-hop_address | interface) [distance_metric]

PSEUDOCODE

For IPv6  addresses, the command is slightly modified:

ipv6 route ipv6_destination_network_prefix (with CIDR) (ipv6_next-hop_address | interface) [distance_metric]

PSEUDOCODE

In both cases, we have a distance_metric parameter. This parameter is optional and defines the cost of using that route. If we have multiple options to reach a destination, then the router will use the lowest cost one. By default, the value of this parameter is 1.

Note that we need to configure routers in both directions to be able to communicate, i.e. if one router knows that it needs to forward packets from one location to another router to reach a destination, the other router also needs to know that it must forward packets to the first router to be able to reach the source. The communication and configuration must be bi-directional. Even pings won’t work without this.

Example

R1(config)#ip route 192.168.2.0 255.255.255.0 s0/1/1
R1(config)#ip route 192.168.2.0 255.255.255.0 192.168.0.1

CLI

R2(config)#ip route 192.168.1.0 255.255.255.0 g0/0
R2(config)#ip route 192.168.1.0 255.255.255.0 192.168.0.6

CLI

Notice that the configuration of both routers is shown, thus allowing bidirectional communication.

## Default Routes

In a routing table, we store entries as destination address and corresponding interface or next hop addresses. However, it is not practical to store an entry for every single possible destination address. Because of this, there is a special entry called a default route. If a destination address cannot be found in the table, the default route is used.

There are two types of default routes:

- Directly Connected Static Default Route – This has just the interface specified.

- Next-Hop Static Default Route – This has the next-hop address specified.

### Primary Static Default Route

There is one more special case of the default route, called the Primary Static Default Route. This specifies the next hop address or interface through which packets from any IP address with any subnet mask will be sent.

The commands to specify a primary static default route for IPv4 and IPv6 respectively are:

ip route 0.0.0.0 0.0.0.0 (next_hop_address | interface) [distance_metric]
ipv6 route ::/0 (ipv6_next_hop_address | interface) [distance_metric]

PSEUDOCODE

## Floating Static Routes

A Floating Static Route is used to forward a packet to a certain destination when the main route is unavailable. We configure a Floating Static Route in the same way that we configure a normal route. The difference is that we give it a higher distance metric, so that it is not normally used.

R1(config)#ip route 192.168.2.0 255.255.255.0 192.168.0.1 5

CLI

Having multiple static routes which have no specified distance metric will cause the router to use one of them randomly.