Routing Information Protocol

The Routing Information Protocol (RIP) is a somewhat old protocol that is no longer used. There are two versions, RIPv1 and RIPv2, and in the few cases where RIP is still used, the latter version is used.

RIPv1 used to flood the network with updates every 30 seconds. This caused a huge amount of overhead for larger networks. RIPv2 overcomes this issue. More information can be found here.

In both versions of RIP, the distance metric used is the hop count. The maximum hop count is set to 15, in order to avoid the looping problem.

## Configuration

Enable RIP:

R1(config)#router rip

CLI

Use RIPv2:

R1(config-router)#version 2

CLI

Set a network to advertise:

R1(config-router)#network 10.0.0.0

CLI

This last command adds a network to the list of networks that will be advertised. Notice how there is no subnet mask. This is because we are using a network address, not a subnetwork address. When we add this network, all interfaces that have an address that belongs to this network will have RIP enabled on them. If one of the interfaces has a subnet address that is under this network address, those will have RIP enabled too. Everything that is one the provided network will now be advertised to other networks that are connected to other interfaces.